## Step04 – Quartz Trace Detection and Slit Geometry Reconstruction

### Purpose

Step04 determines the geometric footprint of every slitlet on the detector using quartz flat-field exposures. The goal is to derive a stable description of the slit geometry that will later be used for:

- slit identification in science frames

- wavelength calibration (Step07)

- rectification of slitlets (Step06)

- optimal spectral extraction (Step08)

The output of Step04 is therefore the reference geometric model of the multi-slit mask on the detector.

The algorithm is designed to be robust to noise, cosmic rays, and imperfect slit illumination, while preserving the detector Y-axis geometry required for wavelength calibration.

# Input Data

Step04 operates on difference images of quartz lamp exposures:

Quartz_B – Quartz_A

for each slit configuration:

- EVEN slits

- ODD slits

The subtraction removes diffuse background and highlights the illuminated slit traces.

Associated information includes:

- detector image

- RA/Dec table describing the targets placed in each slit

- configuration parameters defining segmentation thresholds

# Overview of the Algorithm

The process is composed of four main stages:

- Detect approximate slit centers

- Build a binary slit mask using row-wise segmentation

- Assign slit IDs to mask pixels

- Derive smooth slit geometry models (center + width)

# 1. Slit Center Detection

The first step determines the approximate X position of each slit.

The quartz difference image is collapsed along the spatial direction to produce a 1-D profile:

profile(x) = median_y(diff_image)

Local maxima in this profile correspond to the illuminated slits.

A peak detection algorithm identifies the centroid X position of each slit.

These approximate centers are used only as initial seeds for later segmentation.

The expected number of slits is known from the mask design but is not strictly enforced; the algorithm adapts to the detected peaks.

# 2. Row-Wise Slit Segmentation

Once approximate slit centers are known, the algorithm constructs a binary mask of illuminated slit pixels.

For each slit and each detector row:

- A window centered on the slit center is extracted.

- The local background level is estimated using sidebands outside the slit.

- A detection threshold is computed:

threshold = background + Nσ × local_noise

- Pixels above threshold form candidate slit regions.

- The contiguous region surrounding the slit center is selected.

This procedure yields a row-by-row segmentation of the slit illumination.

Important safeguards include:

- rejection of rows with insufficient signal

- maximum slit width limits

- optional shrinking of edges to avoid contamination

The output is a boolean mask marking slit pixels.

# 3. Slit ID Assignment

Each pixel in the slit mask is assigned to the nearest slit center.

This produces an integer image:

slitid[y,x]

where each pixel contains:

slit ID

or a background value for pixels outside any slit.

This map is used later to:

- identify the slit corresponding to each science spectrum

- measure slit geometry

- support rectification

# 4. Measurement of Slit Geometry

For each slit, the algorithm measures three quantities as a function of detector row:

x_center(y)
x_left(y)
x_right(y)

These are derived from the slitid mask and the quartz image.

The measurements are then smoothed and fitted with robust polynomials.

## Center Trace

The slit center is obtained from the flux-weighted centroid of the illuminated pixels.

The center trace is smoothed and fitted with a polynomial:

x_center(y)

A second-order polynomial is typically sufficient to represent the curvature.

Robust fitting with sigma-clipping suppresses outliers caused by defects or bad rows.

## Slit Width

The slit width is computed as

width(y) = x_right(y) − x_left(y)

Rather than fitting the two edges independently, the current implementation models:

center(y)
width(y)

This ensures that the left and right edges remain parallel and physically consistent.

A low-order polynomial or constant width model is used:

width(y)

## Edge Reconstruction

The model edges are then reconstructed from the center and width:

x_left(y)  = x_center(y) − width(y)/2
x_right(y) = x_center(y) + width(y)/2

This formulation prevents unphysical edge divergence and produces smoother traces.

## Optional Padding

The final edges may be slightly expanded:

x_left  → x_left − pad
x_right → x_right + pad

to ensure the trace fully covers the illuminated slit and allows tolerance for flexure.

# Output Products

Step04 produces three primary outputs for each slit configuration.

## 1. Binary Slit Mask

*_mask.fits

A boolean image identifying illuminated slit pixels.

Used primarily for QC and debugging.

## 2. Slit ID Map

*_slitid.fits

An integer map assigning each pixel to a slit ID.

This map is used downstream for:

- slit extraction

- trace following

- geometry diagnostics

## 3. Geometry File

*_geometry.fits

A multi-extension FITS file containing the geometric model for each slit.

Each extension includes:

- slit ID

- center trace

- left edge trace

- right edge trace

- RA and Dec of the associated target

- polynomial fit coefficients

- diagnostic statistics

This geometry model defines the reference slit coordinate system used in later pipeline steps.

# Relationship with Later Pipeline Steps

Step04 geometry is used in several downstream operations.

### Step06 – Slit Rectification

The geometry model defines the transformation used to straighten the spectra.

### Step07 – Wavelength Calibration

The detector Y coordinate must remain consistent with Step04 so that arc line positions correspond to physical wavelengths.

### Step08 – Optimal Extraction

The slit boundaries define the maximum allowed extraction region, while the ridge detection determines the actual spectrum center.

# Design Principles

The Step04 algorithm follows several key principles.

### Robustness

- row-by-row segmentation

- sigma-clipping fits

- smoothing filters

- fallback solutions for missing data

### Physical Consistency

Edges are derived from center + width rather than fitted independently.

### Stability

Low-order polynomial fits prevent overfitting and maintain smooth geometry.

### Flexibility

The algorithm adapts to both EVEN and ODD slit configurations using the same code path.

# Current Status

The current implementation achieves:

- smooth and stable slit traces

- robust detection across the full detector

- consistent geometry between EVEN and ODD slit sets

A second-order polynomial has been found sufficient to describe the slit curvature while remaining resistant to outliers.