# Step05 — Pixel Flat Construction

## Purpose

Step05 builds the pixel-to-pixel flat-field correction for the detector using the quartz lamp exposures.

Its goal is not to model the slit illumination pattern itself, but to isolate the small-scale detector response variations after removing the large-scale quartz illumination shape.

So this step creates the correction used later to divide science and calibration images by the detector’s pixel-response nonuniformity.

# Main idea

For each slit family, EVEN and ODD, the script constructs:

quartz_diff = Quartz_B - Quartz_A
illum2d     = heavily smoothed quartz_diff inside the slit mask
pixflat     = quartz_diff / illum2d

Then the result is clipped and renormalized so that the median correction is about 1.

In words:

- the quartz difference image gives the observed slit illumination

- a very smoothed version gives the large-scale illumination pattern

- dividing the two leaves the small-scale pixel flat

That is the classic flat-field logic.

# Inputs

The script uses:

### Quartz files

For each trace family:

- QUARTZ_A

- QUARTZ_B_EVEN

- QUARTZ_B_ODD

from config.py

The difference is formed as:

- EVEN: QUARTZ_B_EVEN - QUARTZ_A

- ODD: QUARTZ_B_ODD - QUARTZ_A

### Step04 trace masks

It also uses the slit masks from Step04:

Preferred:

- Even_traces_mask_reg.fits

- Odd_traces_mask_reg.fits

Fallback:

- Even_traces_mask.fits

- Odd_traces_mask.fits

So Step05 depends directly on Step04 having already defined the slit footprint.

# What it does, step by step

## 1. Build the quartz difference image

For each of EVEN and ODD, it loads the two quartz images and subtracts:

quartz = img_b - img_a

This produces the quartz trace image used for the flat-field construction.

## 2. Load the trace mask from Step04

The mask defines where the slit illumination exists on the detector.

Only those masked regions are used to model the illumination and construct the pixel flat. Outside the mask, the flat is left at 1.

## 3. Erode the mask edges

Before smoothing, the script optionally erodes the mask:

mask_er = binary_erosion(mask, iterations=MASK_EROSION_ITERS)

This shrinks the mask inward by a few pixels.

### Why?

Because the slit edges are often less stable and can include thresholding uncertainties or edge effects. By eroding the mask, the smooth illumination model is fit only to the interior of the trace, where the quartz illumination is more reliable.

## 4. Build the smooth 2D illumination model

Inside the eroded mask, the quartz difference image is smoothed very strongly with a 2D Gaussian:

- SIGMA_Y = 75

- SIGMA_X = 25

The smoothing is NaN-aware, so masked-out pixels do not contaminate the result.

This produces:

illum2d

which is the large-scale slit illumination pattern, not the pixel flat itself.

## 5. Divide quartz by the illumination model

The pixel flat is then formed as:

pf = quartz / illum2d

This removes the slowly varying illumination structure and leaves only the high-frequency pixel-to-pixel response variations.

## 6. Clip extreme values and renormalize

Inside the trace mask, the raw flat is:

- clipped between CLIP_LO = 0.5 and CLIP_HI = 2.0

- renormalized by its median

So the final pixel flat has median close to 1 and avoids pathological corrections from bad or low-signal pixels.

Outside the mask, the flat remains exactly 1.

# Outputs

For each trace family, the script writes three files:

### 1. Quartz difference image

{TRACE_BASE}_QuartzDiff_for_pixflat.fits

This is the actual quartz difference used as input to the flat construction.

### 2. Smoothed illumination model

{TRACE_BASE}_Illum2D_quartzdiff.fits

This is the low-frequency 2D illumination pattern derived from the quartz image.

### 3. Pixel flat

{TRACE_BASE}_PixelFlat_from_quartz_diff.fits

This is the final detector pixel-response correction image.

# EVEN and ODD behavior

Unlike some earlier steps, this script processes both sets automatically in one run:

- builds EVEN pixflat

- builds ODD pixflat

So no --set EVEN/ODD argument is needed here. The logic is intentionally dual-set from the start.

# Provenance handling

The script copies useful global provenance from the Step04 mask header into the output headers, such as:

- rotation/flip bookkeeping

- background ID

- slit counts

and it also records:

- which quartz files were used

- which mask was used

- smoothing sigmas

- erosion iterations

- clipping limits

So the output FITS files are reasonably self-documented.

# What Step05 is not doing

It is worth being explicit about this.

Step05 is not:

- rectifying slitlets

- identifying sources

- tracing ridge lines

- deriving wavelength solutions

- correcting large-scale slit throughput variations in science frames directly

It is only building the pixel flat, i.e. the small-scale detector correction.

The large-scale quartz illumination is explicitly modeled and divided out, not preserved.

# One-sentence summary

Step05 uses quartz difference images and the Step04 slit masks to derive a smoothed illumination model and from it build the pixel-to-pixel flat-field correction for both EVEN and ODD slit sets.

If you want, I can also write a short section on possible QC checks for Step05, which would be the natural next thing to document.