# Step06 — Science Frame Construction and Pixel Flat Correction

## Purpose

Step06 produces the final calibrated 2D science frame by applying the pixel flat-field correction derived in Step05 and preparing the data for slit-based extraction and rectification.

This step ensures that detector- and illumination-induced pixel-to-pixel variations are removed, while preserving the flux and geometry of the science exposure.

## Overview of the workflow

Step06 is composed of three sub-steps:

### Step06.0 — Science frame preparation

Script: Step06_00_make_finalscience_PATCHED_rotated.py

- Ingests the reduced science exposure (bias/dark corrected, cosmic-ray cleaned)

- Converts data to physical units (e.g., ADU/s)

- Applies any required image orientation (rotation / flip)

- Produces the baseline product:

FinalScience_*_ADUperS.fits

This frame defines the reference geometry used in all subsequent steps.

### Step06b — Pixel flat-field correction

Script: Step06b_make_finalscience_pixflatcorr_clipped_PATCHED_naming.py

- Applies the pixel flat derived from quartz lamp exposures (Step05)

- Uses the trace mask from Step04 to restrict corrections to illuminated regions

- Optionally clips extreme flat values:

FLCLIPLO ≈ 0.7
FLCLIPHI ≈ 1.3

- Produces:

FinalScience_*_pixflatcorr_clipped_{EVEN|ODD}.fits

#### Key features

- Flat-fielding is applied only inside slit traces

- Background pixels remain untouched (controlled via BKGID)

- Provenance is recorded in headers:

  - SCIENCE, PIXFLAT, MASKFILE

  - clipping parameters

  - trace set (EVEN / ODD)

#### Validation (QC)

- Median(raw/corrected) ≈ 1.0 → flux preserved

- Flat RMS ≈ 4–5% → consistent with detector structure

- No systematic bias introduced

### Step06c — Slit rectification (TRACECOORDS)

Script: Step06c_rectify_slitlets_TRACECOORDS_ONLY_PADX7_PATCHED_naming.py

- Uses trace geometry from Step04 to extract each slit

- Rectifies slit images into trace-aligned coordinates

- Applies padding (PADX ≈ 7 px) to preserve edge pixels

- Outputs a multi-extension FITS (MEF):

  - one HDU per slit: SLIT###

  - includes metadata:

    - slit ID (global EVEN/ODD scheme)

    - RA/Dec

    - polynomial trace model

    - extraction bounds

#### Key design choice

Rectification is done in pixel space only, without wavelength calibration, preserving full flexibility for later spectral alignment.

## Step06 outputs

Main products:

- Full-frame:

  - FinalScience_*_ADUperS.fits

  - FinalScience_*_pixflatcorr_clipped_{EVEN|ODD}.fits

- Rectified slitlets:

  - FinalScience_*_tracecoords_{EVEN|ODD}.fits

## Summary

Step06 transforms the raw science exposure into a flat-fielded, trace-aware, slit-resolved dataset, ensuring:

- accurate pixel response correction

- preserved flux calibration

- consistent geometry for extraction

- compatibility with subsequent wavelength calibration