# Step 08 — Optimal 1D Extraction and Wavelength Attachment

Step 08 converts the rectified, trace-coordinate science frames into calibrated one-dimensional spectra. The process is divided into three stages:

- Step 08.0 — Ridge-guided optimal extraction with sky subtraction

- Step 08.5 — Merge of EVEN and ODD slit sets

- Step 08.6 — Attachment of the global wavelength solution

Together, these steps produce the final extracted spectra for all slits, with associated flux, variance, sky estimate, and wavelength calibration.

# Step 08.0 — Ridge-Guided Optimal Extraction

## Purpose

To extract 1D spectra from rectified slit images using a data-driven ridge tracing and optimal extraction, while performing a robust sky subtraction.

## Inputs

- Rectified science frames in trace coordinates (Step06/06c)

- Slit definitions and geometry

- Exposure-time and variance information (if available)

## Method

### 1. Ridge tracing

For each slit, the algorithm determines the centroid (ridge) position x0(y)along the spatial direction:

- Initialized from the flux distribution (center-seeded)

- Refined row-by-row using local flux maxima or centroiding

- Robust against noise and secondary peaks via corridor constraints

This ridge defines the object trace across the detector.

### 2. Extraction aperture and weighting

An extraction window is defined around the ridge:

- Object aperture: ∣x-x0(y)∣≤wobj

- Optional weighting:

  - uniform or profile-based (approximate optimal extraction)

- Bad pixels and non-finite values are masked

The extracted flux per row is:

F(y)=x∈aperturew(x,y) I(x,y)

### 3. Sky estimation (one-sided pooled sky)

Sky is estimated using pixels on one side of the trace:

- Defined offset from ridge: wsky, in<∣x-x0∣<wsky, out

- Robust estimator (median or clipped mean)

- Pooling across rows to stabilize low-S/N regions

### Important improvement

To avoid contamination from unilluminated regions (e.g., at slit edges), the sky estimate:

- ignores low-signal tails

- excludes non-finite or near-zero pixels

- optionally uses percentile-based trimming

This prevents artificial suppression of sky lines.

### 4. Aperture correction

An aperture correction is applied to account for flux outside the extraction window:

- Derived from cross-dispersion profile

- Applied only when the ridge is sufficiently far from slit edges

- Disabled or limited in edge-truncated cases

### 5. Outputs per slit

Each slit extension contains:

- YPIX — row index (pixel coordinates)

- FLUX — extracted object flux

- VAR — variance estimate

- SKY — sky spectrum

- NOBJ, NSKY — number of contributing pixels

- Ridge and geometry metadata (e.g., X0, flags)

## Behavior at slit edges

At the edges of the field:

- Ridge may approach or cross slit boundaries

- Sky regions may include partially illuminated or empty pixels

- Extraction may be truncated

These cases are handled but flagged; resulting spectra may have:

- reduced S/N

- distorted continuum

- unreliable sky subtraction

# Step 08.5 — Merge EVEN and ODD Sets

## Purpose

To combine spectra extracted separately from EVEN and ODD slit sets into a unified product.

## Inputs

- Step08.0 outputs for:

  - EVEN slits

  - ODD slits

## Method

- Merge all slit extensions into a single FITS file

- Preserve slit identity (SLIT###)

- Ensure consistent ordering and metadata

- Propagate headers and provenance

## Output

A single multi-extension FITS file containing:

- all extracted slits (typically 62 total)

- uniform data structure across slits

# Step 08.6 — Wavelength Solution Attachment

## Purpose

To assign a wavelength scale to each extracted spectrum using the global wavelength calibration derived in Step07.

## Inputs

- Extracted spectra from Step08.5

- Master wavelength solution (Step07.4)

- Slit-dependent shifts relative to master (Step07.5)

## Method

### 1. Pixel-to-detector mapping

Each spectrum is mapped from local row coordinates to detector coordinates:

Ydet=Ypix+Y0DET

where:

- YPIX is the local row index

- Y0DET (or fallback YMIN) is the detector offset

### 2. Alignment to master frame

A slit-dependent shift is applied:

yeff=(Ydet-Ywin,0)+Δshift

where:

- Ywin,0is the master window origin

- Δshift(SHIFT2M) aligns each slit to the master arc

### 3. Wavelength evaluation

The global polynomial solution is applied:

λ(y)=P(yeff)

where Pis typically a 3rd-order polynomial derived from arc lines.

## Outputs

Each slit extension is augmented with:

- LAMBDA_NM — wavelength array (nm)

- Mapping metadata:

  - Y0DET, SHIFT2M

  - transformation description (YMAP)

- Fully calibrated 1D spectra

# Quality and Limitations

## Overall performance

- Wavelength solution is:

  - smooth

  - monotonic

  - consistent across slits

- Typical RMS: ~0.03 nm

## Edge effects

A subset of slits (typically at field edges):

- suffer from partial illumination

- have unstable sky estimates

- may show apparent wavelength misalignment in diagnostic plots

However:

- the underlying wavelength mapping remains valid

- deviations are driven by low S/N and extraction geometry, not calibration errors

# Final Products of Step08

The final output consists of:

- 1D spectra for all slits

- flux, variance, and sky estimates

- wavelength calibration

- full provenance and geometry metadata

These products are ready for:

- scientific analysis

- telluric correction

- line measurements

- coaddition and stacking

# Summary

Step08 performs a complete transformation from rectified 2D slit images to calibrated 1D spectra by:

- Tracing and extracting spectra using a ridge-guided optimal method

- Combining slit sets into a unified dataset

- Applying a global wavelength solution with slit-dependent alignment

The procedure is robust across the field, with well-understood limitations for edge-truncated spectra.