# Step09: Telluric Correction (O₂ A & B Bands)

## Purpose

Step09 removes atmospheric telluric absorption from the extracted spectra, focusing on the molecular oxygen bands:

- B band: ~682–692 nm

- A band: ~752.5–768.5 nm

The correction is performed per slit, using an empirical template derived from the data and a band-specific, weighted fit that accounts for local wavelength offsets and absorption strength.

## Inputs

From Step08 / Step10:

Extract1D_optimal_ridgeguided_POOLSKY_ALL_WAV_OHref.fits

Each slit contains:

- FLUX (or equivalent object spectrum)

- VAR (optional variance)

- LAMBDA_NM

## Processing Overview

Step09 consists of two sub-steps:

# Step09.1 — Build Telluric Template (AB-Decoupled)

### Key improvement

The A-band and B-band templates are now constructed independently, each aligned in its own wavelength region.

### Procedure

For each slit:

- Extract spectral segments:

  - B band: 682–692 nm

  - A band: 752.5–768.5 nm

- Perform local continuum normalization:

  - Linear fit to sidebands on both sides of each band

  - Normalize flux to unity continuum

- Apply quality selection:

  - Sufficient number of pixels

  - Minimum S/N threshold

### Independent alignment (critical)

Each band is aligned separately:

- A-band spectra:

  - aligned using A-band cross-correlation

- B-band spectra:

  - aligned using B-band cross-correlation

This avoids transferring wavelength errors between widely separated regions.

### Stacking

- Median stack of aligned spectra

- Robust interpolation over missing values

### Template construction

For each band:

τ(λ)=-ln⁡(Tmedian(λ))

Output file:

telluric_O2_template_ABdecoupled.fits

Contents:

- O2_ABAND: λ, T(λ), τ(λ)

- O2_BAND: λ, T(λ), τ(λ)

## Key characteristics

- A and B templates are independent

- No assumption of global wavelength rigidity

- Template reflects actual data conditions

# Step09.2 — Apply Telluric Correction (Weighted, Decoupled)

## Major upgrades

Compared to earlier versions, Step09.2 now:

- Decouples A and B bands

- Fits independent wavelength shifts

- Fits independent absorption amplitudes

- Uses weighted least squares emphasizing deep absorption

## Per-slit processing

### 1. Local normalization

For each band independently:

- Fit continuum using sidebands

- Normalize spectrum:

Fnorm=FFcont

### 2. Independent band fitting

For each band (A and B separately):

#### Model

ln⁡Fnorm(λ)=c0-a⋅τ(λ-Δλ)

#### Fit parameters

- Δλ → wavelength shift

- a → absorption amplitude

#### Key feature: weighted fit

Weights are based on template optical depth:

w∝ττmax2

This ensures:

- deep absorption features dominate the fit

- wings and continuum have minimal influence

#### Pixel selection

- only pixels with:

τ>0.05×τmax

### 3. Independent solutions

Each slit gets:

- A-band:

  - TELL_SHA (shift)

  - TELL_AA (amplitude)

  - TELL_OKA

- B-band:

  - TELL_SHB

  - TELL_AB

  - TELL_OKB

### 4. Piecewise correction

Transmission is built independently:

T(λ)=exp⁡(-aA⋅τA(λ-ΔλA))A bandexp⁡(-aB⋅τB(λ-ΔλB))B band1elsewhere

Then:

Fcorr=FT,VARcorr=VART2

## Output

Extract1D_optimal_*_tellcorr_O2_ABdecoupled_weighted.fits

New columns:

- FLUX_TELLCOR_O2

- VAR_TELLCOR_O2 (if variance exists)

## Header keywords (per slit)

| Keyword | Meaning |
| --- | --- |
| TELL_OK | overall success |
| TELL_OKA, TELL_OKB | band-specific success |
| TELL_SHA, TELL_SHB | wavelength shifts (nm) |
| TELL_AA, TELL_AB | amplitudes |
| TELL_A | median amplitude (legacy compatibility) |
| TELL_SH | median shift (legacy compatibility) |
| TELLBAND | bands used (A, B, or A+B) |

## Quality Control

Recommended QC:

- Plot before / after / template per slit (updated QC script)

- Inspect:

  - alignment of deep absorption cores

  - consistency across slits

  - differences between A and B solutions

Important diagnostics:

- TELL_SHA vs TELL_SHB
→ tests wavelength solution consistency

- TELL_AA vs TELL_AB
→ tests continuum / depth consistency

## Known limitations

- Edge slits:

  - incomplete bands

  - unstable continuum normalization

- Low-S/N spectra:

  - unreliable fits (may fallback)

- Template assumptions:

  - single optical-depth scaling per band

  - no modeling of atmospheric variability beyond scaling

## Relationship to other steps

- Step07: global wavelength calibration

- Step08: extraction

- Step09: telluric correction (this step)

- Step10: OH-based wavelength refinement

Step09 corrects absorption, Step10 refines wavelength alignment.

## Practical outcome

Step09 now provides:

- physically consistent telluric correction

- robustness against wavelength non-linearity

- improved removal of both A and B bands

- minimal bias from noisy spectral regions

The resulting spectra are ready for:

- line measurements

- redshift analysis

- flux calibration

- combination across slits