SAMOS pipeline — Step10 summary

Purpose
Step10 refines the wavelength zero-point of the extracted 1D spectra using night-sky OH emission lines. It provides a slit-by-slit fine registration on top of the global wavelength solution derived in Step07 and propagated through Step08.

Input
- Step08 extracted spectra with wavelength solution:
Extract1D_optimal_ridgeguided_POOLSKY_ALL_WAV.fits

- Each slit contains:
FLUX
VAR
SKY
YPIX
LAMBDA_NM
metadata (e.g., Y0DET, SHIFT2M)

Main operations

Step10.1 — Measure OH shifts
- Uses the SKY spectrum in each slit
- Selects multiple OH-rich wavelength windows (~780–930 nm), avoiding the O2 A-band
- Applies a high-pass filter to isolate line structure
- Cross-correlates each slit against a reference slit (auto-selected or specified)
- Measures a constant wavelength shift per slit (in nm)

- For each slit, computes:
- shift_nm (best shift)
- r (correlation coefficient)
- nwin (number of valid windows)
- shift_std_nm (scatter across windows)

- Applies acceptance criteria:
- minimum correlation (min_peak_corr)
- minimum number of windows (min_nwin)
- maximum scatter (max_shift_std_nm)
- maximum absolute shift (max_abs_shift_nm)
- rejection of edge-of-search solutions

- Outputs:
QC_OH_BG_registration.csv

Step10.2 — Apply OH shifts
- Reads the Step10.1 CSV
- Applies a wavelength correction to each slit:

- If:
use = True AND |shift_nm| ≤ clip_nm
→ apply slit-specific shift (source = GOOD)

- Otherwise:
→ apply fallback shift = median of accepted GOOD shifts
(source = FALLBACK)

- No clipped shifts are applied (conservative behavior)

- Updates:
- LAMBDA_NM (shifted wavelength grid)
- header keywords (e.g., OH_SHIFT_NM, SHIFT_SRC)

- Output:
Extract1D_optimal_ridgeguided_POOLSKY_ALL_WAV_OHref.fits

Key characteristics

- Correction is a constant offset per slit (no re-fitting of dispersion)
- Typical shifts are small (~0.0–0.1 nm for good slits)
- Fallback ensures stability for low-S/N or problematic slits
- Step10 is conservative by design and does not override the global calibration

QC interpretation

- A subset of slits (~20–30%) typically yields reliable OH shifts
- Remaining slits fall back to the median correction
- Large measured shifts (>0.5–1 nm) are usually non-physical and rejected
- The fallback median represents the best estimate of residual global offset

Recommended QC metrics
- Number of GOOD vs FALLBACK slits
- Median accepted shift and robust scatter
- Distribution of shift_nm vs slit ID
- Visual OH alignment in representative slits (before/after)

Known limitations

- Edge slits:
- partially illuminated
- unstable sky spectra
- unreliable OH measurements

- Low-S/N slits:
- poor correlation
- insufficient OH features

- Sky subtraction systematics:
- can affect line shapes and correlation quality

These are data-quality limitations, not failures of the wavelength solution.

Relationship to other steps

- Step07:
Provides the global wavelength calibration (polynomial solution)

- Step08:
Extracts spectra and propagates the wavelength solution

- Step09:
Applies telluric correction (e.g., O2 A-band)

- Step10:
Refines wavelength zero-point using OH sky emission

Step10 should be viewed as a fine adjustment layer on top of a validated global calibration.

Practical conclusion

- Step10 improves slit-to-slit wavelength consistency using sky emission lines
- It applies corrections only where robustly measured
- It avoids introducing large or unstable shifts
- The final product is suitable for:
- line measurements
- redshift estimation
- combination of spectra across slits