# Step11 — Flux Calibration and Continuum Correction

## 🎯 Purpose

Step11 converts extracted spectra from instrumental units into physically calibrated flux densities, anchored to broadband photometry and corrected for instrumental and calibration-induced spectral distortions.

It produces the final science-ready spectra:

FLUX_FLAM  [erg s⁻¹ cm⁻² Å⁻¹]

# 🧩 Inputs

### From Step08–10

- Extracted spectra:

  - FLUX, VAR, SKY

  - LAMBDA_NM

- Telluric-corrected spectra:

  - FLUX_TELLCOR_O2

### External calibration data

- Broadband photometry (SkyMapper):

  - r, i, z AB magnitudes

- Instrument throughput table:

  - throughput_total_SAMOS_SOAR_CCD.csv

- Global color correction:

  - global_color_response_correction.csv (derived from data)

# ⚙️ Processing Steps

## 11.1 — Throughput Correction

Remove telescope + instrument + detector response:

FLUX_TPUTCOR = FLUX_TELLCOR_O2 / T_total(λ)

- Uses T_TOTAL (or product of components)

- Variance scaled accordingly

👉 Removes instrumental sensitivity, but not calibration artifacts from flat-fielding

## 11.2 — Global Color Correction (NEW)

Correct continuum distortion introduced by non-flat flat-fielding (quartz lamp imprint).

### Motivation

Flat-fielding with a quartz lamp introduces a multiplicative spectral shape:

F_measured(λ) ∝ F_true(λ) × Lamp(λ)

This imprint is not removed by throughput correction.

### Solution

A global correction curve C(λ)is derived from median photometric residuals across all slits:

C(λ_b) = 10^(-0.4 × Δm_b)

where:

- Δm = m_syn − m_obs

- bands: r, i, z

A smooth function (log–log linear fit) is used to interpolate across wavelength.

### Application

FLUX_COLORCOR = FLUX_TPUTCOR / C(λ)

👉 Removes lamp-induced spectral slope

## 11.3 — Photometric Anchoring

Match spectra to broadband photometry via synthetic photometry.

For each band:

F_syn = ∫ F(λ) T_band(λ) dλ

Compare to observed photometric flux:

Δm = m_syn − m_obs

### Fit model

A low-order correction is applied:

F(λ) → S × F(λ) × (λ/λ₀)^α

where:

- S = global scale

- α = spectral tilt

- λ₀ = pivot wavelength (typically i-band)

Modes:

- gray → S only

- tilt → S + α

### Output

FLUX_FLAM = S × FLUX_COLORCOR × (λ/λ₀)^α

with corresponding variance propagation.

## 11.4 — Output Products

### Per-slit extensions contain:

- FLUX_TELLCOR_O2

- FLUX_TPUTCOR

- FLUX_COLORCOR ← NEW

- FLUX_FLAM ← final calibrated spectrum

and corresponding variance columns.

### Metadata

Headers include:

- TPUTCOR — throughput applied

- COLRCOR — global color correction applied

- FLXPRIOR — calibration mode (gray / tilt)

## 11.5 — Summary Table

Step11_fluxcal_summary.csv contains:

- calibration parameters:

  - S, alpha

- per-band diagnostics:

  - dm_r, dm_i, dm_z

- input photometry:

  - m_r_ab, m_i_ab, m_z_ab

- QC flags:

  - qcflag, nband

# 🔍 Quality Control

## Primary diagnostic

Photometric residuals:

Δm = m_syn − m_obs

### Expected behavior (after full correction)

- median ≈ 0 in all bands

- no trend with wavelength

- scatter ≈ 0.05–0.2 mag

## Secondary diagnostics

- Distribution of alpha

  - should be centered near 0 after color correction

- Spectrum vs photometric anchors

  - consistency, not exact matching

# 🧠 Key Concepts

## Separation of corrections

Step11 cleanly separates three effects:

- Instrument response
→ removed by throughput correction

- Flat-field spectral imprint
→ removed by global color correction

- Absolute flux scale + residual slope
→ set by photometric anchoring

## Why global color correction is necessary

- Quartz lamp ≠ flat spectrum

- Imprints a systematic continuum slope

- Cannot be corrected by throughput alone

- Requires data-driven correction using photometry

## Why not force exact agreement with photometry

Broadband measurements are integrals:

F_band = ∫ F(λ) T(λ) dλ

Therefore:

- spectra should match synthetic photometry

- not pass exactly through plotted points

# 🚀 Final Pipeline Flow (Step09–11)

Step09: Telluric correction
Step10: Wavelength refinement (OH alignment)

Step11:
    1. Throughput correction
    2. Global color correction   ← NEW
    3. Photometric anchoring
    4. Final calibrated spectra

# ✅ Outcome

After Step11:

- spectra are:

  - telluric corrected

  - wavelength calibrated

  - instrument-response corrected

  - continuum-corrected

  - flux calibrated

👉 Ready for:

- line measurements

- continuum analysis

- SED fitting

- scientific publication

# 💡 Notes for Future Work

- The color correction curve can be:

  - reused across runs with same setup

  - refined with more data

- Higher-order corrections are not recommended

- Synthetic photometry accuracy depends on:

  - filter curves

  - wavelength calibration quality

If you want next time, we can:
👉 freeze naming conventions and produce a final “release version” of the pipeline (Steps 06–11)